| ##### Vessel | ##### : |  | ##### MT FRONT CRUISER | ##### Week No | ##### : | ##### 01 | ##### Year: | ##### 2026 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ##### 1 |  | Cargo loading operation at Guana FPSO carried out. |  |  |  |  |  |  |
| ##### 2 |  | Port arrival and departure from Guana FPSO done. |  |  |  |  |  |  |
| ##### 3 |  | A/E monthly performance taken & recorded for all3 Aux Engines. |  |  |  |  |  |  |
| ##### 4 |  | A/E # 3 L.O filter routine done. |  |  |  |  |  |  |
| ##### 5 |  | A/E spare fuel valves overhauled. |  |  |  |  |  |  |
| ##### 6 |  | Fresh water generator boxed back and put on use. |  |  |  |  |  |  |
| ##### 7 |  | OWS feed water P/P filter routine done. |  |  |  |  |  |  |
| ##### 8 |  | OWS operated to pump out ER bilge water. |  |  |  |  |  |  |
| ##### 9 |  | No #2 COPT laggings put back and tried out. |  |  |  |  |  |  |
| ##### 10 |  | Spare motors shifted and secured at Various locations in ER. |  |  |  |  |  |  |
| ##### 11 |  | Sluge P/P filter routine done |  |  |  |  |  |  |
| ##### 12 |  | BWTS Tried out in Ballasting mode. |  |  |  |  |  |  |
| ##### 13 |  | Weekly safety routine test and checks carried out. |  |  |  |  |  |  |
| ##### 14 |  | Weekly onboard lube oil test done. |  |  |  |  |  |  |
| ##### 15 |  | Boiler water test carried out. |  |  |  |  |  |  |
| ##### 16 |  | ER blower flaps and funnel flaps general greasing carried out. |  |  |  |  |  |  |
| ##### 17 |  | Engine room general cleaning and painting done. |  |  |  |  |  |  |
|  |  | Electrical Work done |  |  |  |  |  |  |
| 1 |  | AVSS #1 Discharge valve Smart Positioner renewed |  |  |  |  |  |  |
| 2 |  | Bridge CCTV PC backup UPS battery renewed |  |  |  |  |  |  |
| 3 |  | BWTS electrolyser A1 No.4 Negative terminal insulation guide leak rectified |  |  |  |  |  |  |
| 4 |  | Engine Room Ventilation Blower Remote panel troubleshoot in progress |  |  |  |  |  |  |
| 5 |  | M.E Shaft Earthing Slip ring and carbon brush cleaning carried out |  |  |  |  |  |  |
| 6 |  | Pump Room Fixed Gas detection system Alarm Strobe light Circuit renewed |  |  |  |  |  |  |
| 7 |  | Ships SPECVISION PC troubleshoot in progress as per the makers instruction |  |  |  |  |  |  |
| 8 |  | Monthly motor greasing carried out. |  |  |  |  |  |  |
| 9 |  | ME pickup sensor inspected and cleaned. |  |  |  |  |  |  |
| 10 |  | Saturday routines & alarms test carried out, found satisfactory & life boat engine tried out on both the batteries found satisfactory. |  |  |  |  |  |  |
| 11 |  | Shaft earthing slipring cleaned and general inspection carried out. |  |  |  |  |  |  |
| 12 |  | Saturday routines & alarms test carried out, found satisfactory & life boat engine tried out on both the batteries found satisfactory. |  |  |  |  |  |  |